B. Tech Project Report

On

Investigation of Dislocation Density of Directed Energy Deposited 316L Steel at Elevated Temperatures.

BY

Jaya Vardhan

Discipline of Metallurgical Engineering and Materials Science

Indian Institute of Technology Indore

2024

Investigation of Dislocation Density of Directed Energy Deposited 316L Steel at Elevated Temperatures.

A PROJECT REPORT

Submitted in partial fulfilment of the

requirements for the award of the degrees

of

BACHELOR OF TECHNOLOGY

in

Metallurgical Engineering and Materials Science

Submitted by:

Jaya Vardhan

Guided by:

Dr. Eswara Prasad Korimilli, Associate Professor, Dept. of MEMS, IIT Indore

Indian Institute of Technology Indore

November, 2024

CANDIDATE’S DECLARATION

I hereby declare that the project entitled “Investigation of Dislocation Density of Directed Energy Deposited 316L Steel at Elevated Temperatures.” submitted in partial fulfilment for the award of the degree of Bachelor of Technology in ‘Metallurgical Engineering and Materials Science’ completed under the supervision of Dr. Eswara Prasad Korimilli, Associate Professor, Dept. of MEMS, IIT Indore is an authentic work.

Further, I declare that I have not submitted this work for the award of any other degree elsewhere.

Signature and name of the student with date

CERTIFICATE by BTP Guide

It is certified that the above statement made by the students is correct to the best of my knowledge.

Signature of BTP Guide with dates and their designation

Preface

This report on “Investigation of Dislocation Density of Directed Energy Deposited 316L Steel at Elevated Temperatures” is prepared under the guidance of Dr. Eswara Prasad Korimilli.

This project, titled "Investigation of Dislocation Density of Directed Energy Deposition 316L Steel at Elevated Temperatures," marks the culmination of my undergraduate journey in metallurgy and materials science. The research focuses on understanding the microstructural behaviour of 316L steel produced by Additive Manufacturing, specifically Directed Energy Deposition (DED), under various thermal conditions. My work involved experimental techniques such as X-ray Diffraction (XRD) and Thermogravimetric Analysis (TGA) to study residual stresses and dislocation densities.

This project has not only deepened my technical knowledge in metallurgy but also enhanced my analytical and problem-solving skills. It has been an invaluable learning experience, preparing me for future endeavours in research and industry. I hope this report provides insights into the potential of additive manufacturing in producing advanced materials for industrial applications.

Jaya Vardhan

B.Tech. IV Year

Discipline of Metallurgical Engineering and Materials Science

IIT Indore

Acknowledgements

I would like to thank my project guide, Dr. Eswara Prasad Korimilli, for his guidance and support during the course of this project. His feedback and suggestions have been helpful in completing this work.

Jaya Vardhan

B.Tech. IV Year

Discipline of Metallurgical Engineering and Materials Science

IIT Indore

Abstract

The advancement of additive manufacturing technologies, such as Directed Energy Deposition (DED), has significantly transformed the field of material fabrication by enabling the production of complex, geometrically intricate metal components with high precision. Among the materials commonly used in DED, 316L stainless steel stands out due to its excellent corrosion resistance, mechanical strength, and suitability for high-temperature applications, making it a preferred choice for industries such as aerospace, automotive, and biomedical.

Despite its many advantages, the DED process introduces challenges related to the development of residual stresses and dislocation density within the material. These internal stresses and defects can greatly influence the material’s mechanical behaviour, particularly its performance under thermal cycling or at elevated temperatures. The formation of residual stresses, due to rapid heating and cooling during the DED process, can lead to warping, distortion, or even failure of components in critical applications. Similarly, dislocation density, which reflects the amount of crystal imperfections, can significantly affect the material’s strength and ductility.

This project aims to investigate the residual stress and dislocation density in 316L stainless steel fabricated via DED, with a particular focus on understanding how these factors evolve at various elevated temperatures. The research is centred around the application of X-ray Diffraction (XRD) techniques to measure the residual stresses and estimate the dislocation density at different stages of thermal exposure. By analysing these microstructural characteristics, the project seeks to offer insights into how temperature variations during the DED process influence the material’s mechanical properties, such as hardness, strength, and overall performance.

The study not only aims to contribute to the understanding of the fundamental mechanisms driving these changes but also seeks to provide valuable data for optimizing the DED process. By identifying the thermal conditions that lead to minimal residual stress and dislocation density, this research has the potential to enhance the quality and reliability of 316L stainless steel components produced through additive manufacturing. Additionally, the findings of this project could help inform the development of more effective post-processing techniques and better predict the material's behaviour in real-world applications.

Table of Contents

Candidate’s Declaration

Supervisor’s Certificate

Preface

Acknowledgements

Abstract

Chapter 1. Introduction

  - 316 L Steel

  - Additive Manufacturing

  - Directed Energy Deposition

  - Thermogravimetric Analysis

  - Bragg’s Law

  - X-Ray Diffraction

Chapter 2. Literature Review

2.1 XRD Peak Broadening

2.2 Curve Fitting

2.3 Relation between Strain and Crystallite Size

2.4 Dislocation Density

Chapter 3. Experimental Procedure

Chapter 4. Results

4.1 TGA

4.2 Determining Crystal Structure from X-Ray Diffractogram

4.3 X-Ray Diffractograms at Elevated Temperatures

4.4 Dislocation Density Calculations

4.5 Microstructures

Chapter 5. Analysis

5.1 Peak Shift v/s Temperature

5.2 Lattice Parameter v/s Temperature

5.3 Grain Size v/s Temperature

5.4 Strain v/s Temperature

5.5 Dislocation Density v/s Temperature

5.6 Crystallite Size v/s Temperature

5.7 Hardness v/s Temperature

List Of Figures

List Of Tables

Chapter 1

Introduction

1.1 316L Steel

Type 316L stainless steel is a molybdenum-bearing austenitic. It is more resistant to general corrosion and pitting than conventional nickel chromium stainless steels such as 302-304. L-grades have 0.03% carbon maximum. L-grades are resistant to sensitization in short-term exposures or heat treatments. L-grade often has slightly lower strength than standard stainless steel [1].

Adding molybdenum significantly improves the resistance of 316L to pitting and crevice corrosion in chloride-containing environments.

Fig. 1: Composition of 316L steel [1]

1.2 Additive Manufacturing

Additive manufacturing (AM), also known as 3D printing, is a process used to fabricate a physical object from a three-dimensional (3D) digital model, typically by laying down and bonding many successive thin layers of materials. A few advantages and disadvantages of this process are listed in Table 1.

Table 1: Advantages and Disadvantages of Additive Manufacturing [2]

| Advantages | Disadvantages |
| --- | --- |
| Rapid Process | High Initial Costs |
| Design Freedom | Size Constraints |
| Streamlined workshops | Material Limitations |

There are several types of AM processes, each suitable for different applications, materials, and industries. They are:

- Material Jetting

- VAT Photo polymerization

- Binder Jetting

- Material Extrusion

- Directed Energy Deposition

- Powder Bed Fusion

- Sheet Lamination

1.3 Directed Energy Deposition

Directed Energy Deposition (DED) processes direct energy into a narrow, focused region to heat a substrate, melting the substrate and simultaneously melting material that is being deposited into the substrate’s melt pool. The energy source can be an electron beam or a laser beam. The feedstock can be in the form of powder or wire.

Figure 2 illustrates the process of laser-based powder Directed Energy Deposition (DED).

Fig. 2: Laser based powder DED [2]

“melt materials as they are being deposited” is the core principle of DED. Note that the energy density of the beam must be above a critical value to form a melt pool on the substrate.

1.4 Thermogravimetric Analysis

Thermogravimetric Analysis (TGA) is a technique in which the mass of a substance is monitored as a function of temperature and time as the sample specimen is subjected to a controlled temperature program in a controlled atmosphere.

Fig. 3: Working Principle of TGA [3]

Figure 3 illustrates the working principle of TGA. The heart of the thermogravimetric analyser is the “balance,” which can measure the sample mass as a function of temperature and time. Heating parameters are controlled using “Furnace programmer.” Computers are used for data acquisition.

1.5 Bragg’s Law

Diffraction: Diffraction is a scattering phenomenon which occurs whenever wave motion encounters a set of regularly spaced scattering objects, given that the wavelength of the wave motion is of the same order of magnitude as the repeat distance between the scattering centres [4]. Figure 4 shows the diffraction phenomenon of X-rays by a crystal.

Fig. 4: Diffraction of X-rays by a crystal [4]

When an X-ray encounters an atom, it gets scattered in all directions. In figure 4, only those diffracted beams are shown whose exit angle is equal to the angle of incidence. In plane A, rays 1 and 1a are striking K and P atoms respectively. After striking, they get scattered in all directions but only 1' and 1a' are completely in phase and can reinforce each other. The path difference between 1' and 1a' is QK-PR.

QK -PQ=PKCosθ -PKCosθ=0

Therefore, the path difference between 1' and 1a' is zero. This implies that they are in same phase.

Similarly, considering ray 1 and ray 2, the path difference is ML+LN. This is the additional distance travelled by ray 2.

ML+LN=d'Sinθ+d'Sinθ   (where d'is the interplanar spacing)

It is the constructive interference of scattering from the atoms which produces diffracted intensity. Constructive interference occurs when the path difference between the two waves is an integer multiple of the wavelength (𝜆).

Therefore,                          Path difference=nλ       (where n = 1,2,3…)

⇒nλ = 2d'Sinθ  Bragg's Law

1.6 X-Ray Diffraction

XRD is a high-precision non-destructive technique that provides important information about the structural characteristics of the materials by enabling chemical composition identification [5].

Every crystalline substance gives a pattern; the same substance always gives the same pattern; and in a mixture of substances, each produces its pattern independently of the others – Hull (1919).

Working Principle:

From Figure 5, X-rays from the tube T are incident on a crystal C which may be set at any desired angle to the incident beam by rotation about an axis through O. D is a detector which measures the intensity of the diffracted X-rays.

Fig. 5: Schematic of XRD [4]

Scattering of an X-Ray by a single electron:

Ir= I0K r2 Sin2α

Where,  r is the distance from electron

α is the scattering angle

Scattering of an X-Ray by an atom:

When an x-ray beam encounters an atom, each electron in it scatters part of the radiation coherently in accordance with the Thomson equation [4].

f= Amplitude of the wave scattered by an atomAmplitude of the wave scattered by an electron

f  is known as “the atomic scattering factor.” It is used to describe the efficiency of scattering of a given atom in a given direction.

Structure Factor:

Structure Factor is a mathematical expression used to describe how X-rays are scattered by the atoms in a crystal lattice. It provides critical information about the distribution of atoms within the crystal and is essential in determining the crystal’s atomic structure from X-ray diffraction data [4].

Fhkl= Σ fn . e2πihun+kvn+lwn

where,     F is the Structure Factor.

fn is the Atomic Scattering Factor of  nth atom.

h k l are the Miller Indices.

un vn wn are the co-ordinates of the position of  nth atom.

F= Amplitude of the wave scattered by all the atoms of a unit cellAmplitude of the wave scattered by one electron

The intensity of the beam diffracted by all atoms of the unit cell in a direction predicted by Bragg’s law is proportional to F2.

I ∝F2

Structure Factor for FCC unit cell:

The FCC unit cell contains 4 atoms. These 4 atoms are positioned at the following fractional coordinates within the unit cell. They are:

0 , 0 , 0  -  Corner Atom.

12 , 12, 0  -  Face-centered atom on the XY Plane.

12 ,  0,  12   -  Face-centered atom on the XZ Plane.

0 ,  12,  12   -  Face-centered atom on the YZ Plane.

Substituting these values in the previously stated Structure Factor equation yields:

(Fhkl)FCC =f (e2πih0 + k0 + l0+e2πih12 + k12 + l0 +e2πih12 + k0 + l12  +e2πih0 + k12 + l12 )

⇒(Fhkl)FCC  =f (1 +eπih + k +eπih + l  +eπik + l )

From this equation, it can be stated that:

Case 1: h + k + l is even i.e. h, k, l are all even or all odd (unmixed indices)

(Fhkl)FCC =4f

Case 2: h + k + l is odd (mixed indices)

(Fhkl)FCC =0

Therefore, reflections may occur for such planes as (111), (200), (220) etc.

Chapter 2

Literature Review

2.1 XRD Peak Broadening

Paterson [6] built upon the study conducted by Warren and Averbach [7], identifying three key factors contributing to the broadening of XRD peaks.

- Size Broadening

- Strain Broadening

- Instrument Broadening

Size Broadening:

The size broadening of an XRD peak is determined by “Scherrer’s Equation.” It states:

βsize= KλD Cosθ

Where,  D - Average Crystallite Size.

λ  - Incident radiation wavelength.

βsize -  peak broadening due to size

θ  -  Half of diffraction angle.

K  -  Shape Factor (K = 0.9 for spherical particles)

Strain Broadening:

Differentiating Bragg’s law yields the relation between the strain and strain broadening [4].

βstrain=2ε Tanθ 48

The derivation for this equation is stated below.

Bragg's Law: nλ = 2dSinθ  →1

In this case, strain is defined as ε = Δdd

For a change in the interplanar spacing (Δd), there is a corresponding change in the Bragg angle (Δθ).

nλ = 2d+ΔdSinθ+Δθ

Expanding Sin (A + B) = SinACosB + CosASinB

⇒nλ = 2d+ΔdSinθ.CosΔθ+SinΔθ.Cosθ

Assuming Δθ is small, CosΔθ=1 and SinΔθ=Δθ

⇒nλ = 2d+ΔdSinθ+Δθ.Cosθ

⇒nλ = 2(dSinθ+dΔθCosθ+ΔdSinθ+ΔdΔθCosθ)

But, ΔdΔθ ≈0

Therefore,  nλ = 2(dSinθ+dΔθCosθ+ΔdSinθ)  →(2)

Subtracting 2 and 1 gives ;

nλ = 2dSinθ+2dΔθCosθ+2ΔdSinθ

-           nλ = 2dSinθ

2dΔθCosθ+2ΔdSinθ=0

⇒Δθ=-Δdd Tanθ =- ε Tanθ    (negative sign indicates direction of peak shift)

Peak Broadening due to strain  βstrain=2Δθ=2Tanθ

Instrument Broadening:

This refers to the additional broadening of diffraction peaks caused by imperfections and limitations of the instrument itself, rather than the sample.

Factors contributing to Instrument Broadening are:

- Detector resolution: Limited resolution of the detector used to measure the diffraction pattern can contribute to peak broadening.

- Crystal imperfections in optics: Imperfections in monochromators or focusing crystals within the X-ray instrument can distort the beam and broaden peaks.

- Instrument vibrations: Mechanical vibrations within the instrument can also cause broadening.

- X-ray source size: The finite size of the X-ray source can lead to divergence, which affects the resolution and broadens the peaks.

2.2 Curve Fitting

Curve fitting is a process of finding a mathematical function that best represents a set of data points.

Curve fitting of an X-ray diffractogram is crucial for accurate peak identification and quantitative analysis, allowing for precise localization of peaks and extraction of key data like intensity and area. It also reduces noise, enhancing the clarity of meaningful features in the data.

The Gaussian function is often the best choice for fitting an X-ray diffractogram due to its ability to accurately represent the shape of diffraction peaks, which are typically symmetrical and bell-shaped. The bell-shape nature of the gaussian curve is illustrated in Figure 6.

Fig. 6: Bell Shape of Gaussian Function [9]

Gaussian Function: yx=A . exp-x-xo22σ2 [9]

Where,  A – Peak Height (Amplitude).

σ – Standard deviation.

xo – Peak Position.

Full Width Half Maximum (FWHM) occurs at yx= A2

Substituting this and solving yields  FWHM=22 ln2 σ

2.3 Relation between Strain and Crystallite Size

Since Gaussian broadening combines quadratically, the relationship becomes additive in terms of their squares. This means that the overall broadening effect can be expressed as the sum of the squares of the individual contributions.

βexp2=βsize2+βstrain2+βinst2 4

βexp2-βinst2= Kλ2DCosθ2+ 2ε Tanθ2

βexp2-βinst2Cos2θ= 4ε2Sin2θ + Kλ2D2 →1

1 is in the format⇒y=mx+c

Where, y=βexp2-βinst2Cos2θ

x=4Sin2θ

m=ε2

c = Kλ2D2

Therefore, Strain and Crystallite size can be calculated from slope and intercept respectively.

2.4 Dislocation Density

Chapter 3

Experimental Procedure

Sample Preparation: 1300W_DED_316L steel samples were cut using an Electric Discharge Machine (EDM).

Polishing: The samples were then polished using a Disc Polishing Machine to obtain a smooth surface for subsequent analyses.

TGA: To assess the thermal stability and potential weight loss of the samples at elevated temperatures, TGA was conducted. This step provided an insight into the thermal degradation properties of the material.

XRD: XRD measurements were performed on the samples to analyse their crystallographic structure. The samples were heated to 400°C, 600°C, 800°C, 1000°C, and 1150°C, and after reaching each temperature, XRD readings were recorded. An initial reading was taken at 25°C for all samples before heating.

Plotting: All plots were generated using Origin software. [10]

Experimental Parameters

Incident radiation wavelength – Cu Kα1 = 1.5406 Å

Scan Mode – Continuous

Scan Rate – 300 scans per minute

Sensitivity in 2θ measurement – ±0.01° = ±0.00017 Radians

X-ray Tube parameters –

Cathode – Tungsten; Anode – Copper; Voltage – 45kV; Current – 30mA

Sample Stage – Platinum heat strip (direct heating)

Thermocouple – Pt 10% RhPt

βinst=0.05°

Chapter 4

Results

4.1 TGA

Thermogravimetric Analysis (TGA) was conducted prior to high-temperature X-ray Diffraction (XRD) to assess any potential mass loss or melting of the sample. As shown in Figure 7, the TGA thermal curve displayed an initial dip, attributed to the removal of porosity within the material, followed by a stable ascend. The absence of significant weight loss throughout the analysis confirmed that no mass loss occurred, indicating the sample's readiness for subsequent high-temperature XRD measurements.

Fig. 7: TGA of DED 316L

4.2 Determining Crystal Structure from X-Ray Diffractogram

Figure 8 shows the X-ray diffractogram of DED 316L steel at 25°C. The peak positions were determined using Gaussian curve fitting, with the results summarized in Figure 8. The interplanar spacing (d) values were calculated using Bragg’s law, as explained in Section 1.5. 1d2 ratios are calculated for all 3 peaks and are rationalised to convert them into whole numbers. The values are listed in Table 2.

Fig. 8: XRD of DED 316L at 25°C

| Peak No. | d-Spacing (Å) | 1/d2 | Ratio | Ratio * 2 | Ratio * 3 |
| --- | --- | --- | --- | --- | --- |
| 1 | 2.06 | 0.24 | 1.00 (Peak 1 / Peak 1) | 2.00 | 3.00 |
| 2 | 1.79 | 0.31 | 1.32 (Peak 2 / Peak 1) | 2.64 | 3.98 |
| 3 | 1.27 | 0.64 | 2.67 (Peak 3 / Peak 1) | 5.34 | 8.01 |

Table 2: Interplanar spacing ratios of DED 316L at 25°C

For an FCC Crystal Structure, the theoretical ratio of the lattice planes (h k l) follows the sequence:

1st Peak (1 1 1) – h2+ k2+ l2=3

2nd Peak (2 0 0) – h2+ k2+ l2=4

3rd Peak (2 2 0) – h2+ k2+ l2=8

From theory, the ratio of first 3 peaks of FCC is 3 : 4 : 8.

From Table 2, the ratio of final values is 3 : 3.98 : 8.01.

Therefore, experimental ratio is approximately equal to the theoretical ratio. This implies that the crystal structure of 316L steel is FCC.

4.3 X-Ray Diffractograms at Elevated Temperatures

XRD at 400°C

Fig. 9: Heating Cycle at 400°C

Fig. 10: XRD at 400°C

At 25°C                                			At 400°C

Table 3: Peak Position and FWHM at 400°C

XRD at 600°C

Fig. 11: Heating Cycle at 600°C

Fig. 12: XRD at 600°C

At 25°C                                                            At 600°C

Table 4: Peak Position and FWHM at 600°C

XRD at 800°C

Fig. 13: Heating Cycle at 800°C

Fig. 14: XRD at 800°C

At 25°C                                                            At 800°C

Table 5: Peak Position and FWHM at 800°C

XRD at 1000°C

Fig. 15: Heating Cycle at 1000°C

Fig. 16: XRD at 1000°C

At 25°C                                                            At 1000°C

Table 6: Peak Position and FWHM at 1000°C

XRD at 1150°C

Fig. 17: Heating Cycle at 1150°C

Fig. 18: XRD at 1150°C

At 25°C                                                            At 1150°C

Table 7: Peak Position and FWHM at 1150°C

4.4 Dislocation Density Calculations

βexp2-βinst2Cos2θ   v/s   4ε2Sin2θ  plots are demonstrated at different temperatures with their corresponding plot at room temperature. Using the equation derived in Section 2.3 and Section 2.4, Crystallite Size, Strain and Dislocation Densities are calculated using the Peak Positions and FWHM values mentioned in Section 4.3.

βexp2-βinst2Cos2θ= 4ε2Sin2θ + Kλ2D2

δ=Kε2b2

- 400°C

- (b)

Fig. 19: (a) At 25°C  (b) At 400°C

|  | Crystallite Size (nm) | Strain | Dislocation Density (m-2) |
| --- | --- | --- | --- |
| 25°C | 36 | 6.30*10-3 | 1.01*1016 |
| 400°C | 31.5 | 5.43*10-3 | 7.42*1015 |

Table 8: Crystallite Size and Dislocation Density at 400°C

- 600°C

- (b)

Fig. 20: (a) At 25°C  (b) At 600°C

|  | Crystallite Size (nm) | Strain | Dislocation Density (m-2) |
| --- | --- | --- | --- |
| 25°C | 42 | 5.99*10-3 | 9.11*1015 |
| 600°C | 30.9 | 5.40*10-3 | 7.26*1015 |

Table 9: Crystallite Size and Dislocation Density at 600°C

- 800°C

- (b)

Fig. 21: (a) At 25°C  (b) At 800°C

|  | Crystallite Size (nm) | Strain | Dislocation Density (m-2) |
| --- | --- | --- | --- |
| 25°C | 38.4 | 6.37*10-3 | 1.03*1015 |
| 800°C | 30.2 | 5.38*10-3 | 7.15*1015 |

Table 10: Crystallite Size and Dislocation Density at 800°C

- 1000°C

- (b)

Fig. 22: (a) At 25°C  (b) At 1000°C

|  | Crystallite Size (nm) | Strain | Dislocation Density (m-2) |
| --- | --- | --- | --- |
| 25°C | 37 | 6.31*10-3 | 1.01*1015 |
| 1000°C | 31.1 | 5.36*10-3 | 7.37*1015 |

Table 11: Crystallite Size and Dislocation Density at 1000°C

- 1150°C

- (b)

Fig. 23: (a) At 25°C  (b) At 1150°C

|  | Crystallite Size (nm) | Strain | Dislocation Density (m-2) |
| --- | --- | --- | --- |
| 25°C | 41.4 | 6.01*10-3 | 9.19*1015 |
| 1150°C | 40.9 | 5.32*10-3 | 6.91*1015 |

Table 12: Crystallite Size and Dislocation Density at 1150°C

4.5 Microstructures

(a)                                                                                      (b)

(c)                                                                                      (d)

(e)

Fig. 24: (a) At 400°C (b) At 600°C (c) At 800°C (d) At 1000°C (e) At 1150°C

| Temperature | Grain Diameter (μm) |
| --- | --- |
| 400°C | 5.93 |
| 600°C | 6.93 |
| 800°C | 7.48 |
| 1000°C | 12.45 |
| 1150°C | 8.82 |

Table 13: Grain Diameter at various Temperatures

Chapter 5

Analysis

5.1 Peak Shift v/s Temperature

Figure 25 illustrates the peak shift variation with temperature for (111), (200), and (220) peaks of DED_316L. The peak shift is calculated by considering the peak position of 25°C calculated for that respective sample as reference.

Fig. 25: (a) (111) Peak

Fig. 25: (b) (200) Peak

Fig. 25: (c) (220) Peak

Observations

- Peak Shift increases with increasing peak number at const. temperature.

- Peak Shift increases with increasing temperature till 1000°C for any peak.

Reason for Peak Shift at elevated temperatures:

A crystal does not have its atoms arranged on a perfectly regular lattice extending from one side of the crystal to the other. Instead, the lattice is broken up into several tiny blocks, each slightly disoriented from one another. This is demonstrated in figure 17. Let’s say “𝛼” is the maximum disorientation angle.

Fig. 26: Representation of an actual lattice showing disorientation.

If “𝜃” is the incident beam angle, diffraction occurs around “𝜃+ 𝛼.” [4]

“𝛼” decreases at elevated temperatures due to “polygonization,” thus causing peak shift towards left side.

5.2 Lattice Parameter v/s Temperature

From Bragg’s Law,

d= λ2Sinθ

a=dh2+ k2+l2

where a is lattice parameter

The lattice parameters for all three peaks were calculated using the described method, and their average values are presented in Figure 27.

Fig. 27: Lattice Parameter v/s Temperature

Observations

- Lattice Parameter increases with increasing temperature.

5.3 Grain Size v/s Temperature

From the values reported in Section 4.5, Figure 28 illustrates the variation of grain diameter with temperature.

Fig. 28: Grain Diameter v/s Temperature

5.4 Strain v/s Temperature

From the values reported in Section 4.4, Figure 29 illustrates the variation of strain with temperature.

Fig. 29: Strain v/s Temperature

Observations

- Strain decreases with increasing temperature due to the relief of internal stresses.

5.5 Dislocation Density v/s Temperature

From the values reported in Section 4.4, Figure 30 illustrates the variation of dislocation density with temperature.

Fig. 30: Dislocation Density v/s Temperature

Observations

- Dislocation density decreases with increasing temperature.

Dislocation density decreases during high-temperature annealing because the heat provides energy for atoms to move, allowing dislocations to rearrange, annihilate, or merge. This process reduces the overall dislocation count, leading to the recovery of the material, resulting in a lower dislocation density.

5.6 Crystallite Size v/s Temperature

From the values reported in Section 4.4, Figure 31 illustrates the variation of dislocation density with temperature.

Fig. 31: Crystallite Size v/s Temperature

5.7 Hardness v/s Temperature

Fig. 32: Hardness v/s Temperature

Observations

- Strain decreases with increasing temperature due to decrease in dislocation density.

Hardness decreases with a reduction in dislocation density because dislocations are obstacles to the movement of other dislocations. A high dislocation density means more barriers, making the material harder and stronger. When dislocation density decreases, fewer obstacles are present, allowing atomic planes to slip more easily under stress, resulting in reduced hardness.